SECTION 083402

CROWDER CANTILEVER SLIDING DOOR HARDWARE

GENERAL

GENERAL PROVISIONS

Attention is directed to the CONTRACT AND GENERAL CONDITIONS and all Sections within DIVISION 01 - GENERAL REQUIREMENTS which are hereby made a part of this Section of the Specifications.

DESCRIPTION OF WORK

Work Included:  Provide labor, materials and equipment necessary to complete the work of this Section, including but not limited to the following:

Cantilever sliding door tracks

Catch ‘N’ Close hardware

Related Work: The following items are not included in this Section and are specified under the designated Sections:

Section 087100 - Door Hardware for door hardware not specified in this Section.

Section 087111 – Exposed Track and Hardware Sliding Door Systems.

Section 087113 – Under Glass Sliding Door Systems.

Section 087114 – Pocket Door Hardware.

REFERENCE STANDARDS

Comply with the following reference standards and publications as applicable to the project..

American National Standards Institute (ANSI):

ANSI A117.1  Accessible and Usable Buildings and Facilities.

Door and Hardware Institute (DHI):

ANSI/DHI A115.IG  Installation Guide for Doors and Hardware.

DHI Sequence and Format for the Hardware Schedule.

SUBMITTALS

Product Data:  Include installation details, material descriptions, dimensions of individual components and profiles, and finishes.

Door Hardware Schedule:  Prepared by or under the supervision of supplier, detailing fabrication and assembly of door hardware, as well as procedures and diagrams.  Coordinate the final Door Hardware Schedule with doors, frames, and related work to ensure proper size, thickness, hand, function, and finish of door hardware.

Format:  Comply with scheduling sequence and vertical format in DHI's "Sequence and Format for the Hardware Schedule."

Organization:  Organize the Door Hardware Schedule into door hardware sets indicating complete designations of every item required for each door or opening.

Organize door hardware sets in same order as in the Door Hardware Schedule at the end of Part 3.

Content:  Include the following information:

Type, style, function, size, label, hand, and finish of each door hardware item.

Manufacturer of each item.

Fastenings and other pertinent information.

Location of each door hardware set, cross-referenced to Drawings, both on floor plans and in door and frame schedule.

Explanation of abbreviations, symbols, and codes contained in schedule.

Mounting locations for door hardware.

Door and frame sizes and materials.

Submittal Sequence:  Submit the final Door Hardware Schedule at earliest possible date, particularly where approval of the Door Hardware Schedule must precede fabrication of other work that is critical in the Project construction schedule.  Include Product Data, Samples, Shop Drawings of other work affected by door hardware, and other information essential to the coordinated review of the Door Hardware Schedule.

Qualification Data:  For firms and persons specified in "Quality Assurance" Article.

Include lists of completed projects with project names and addresses of architects and owners, and other information specified.

Maintenance Data:  For each type of door hardware to include in maintenance manuals specified in Division 01.

Warranties:  Special warranties specified in this Section.

QUALITY ASSURANCE

Installer Qualifications:  An experienced installer who has completed door hardware similar in material, design, and extent to that indicated for this Project and whose work has resulted in construction with a record of successful in-service performance.

Supplier Qualifications:  Door hardware supplier with warehousing facilities in Project's vicinity and who is or employs a qualified Architectural Hardware Consultant, available during the course of the Work to consult with Contractor, Designer, and the Owner about door hardware and keying.

Architectural Hardware Consultant Qualifications:  A person who is currently certified by the Door and Hardware Institute as an Architectural Hardware Consultant and who is experienced in providing consulting services for door hardware installations that are comparable in material, design, and extent to that indicated for this Project.

Source Limitations:  Obtain each type and variety of door hardware from a single manufacturer, unless otherwise indicated.

Regulatory Requirements:  Where indicated to comply with accessibility requirements:

- Editor’s Note: Insert local regulatory requirements as needed.

[Comply with local regulatory requirement]

Comply with Americans with Disabilities Act (ADA), "Accessibility Guidelines for Buildings and Facilities (ADAAG)."

Preinstallation Conference:  Conduct conference at Project site to comply with requirements in Division 01. Review methods and procedures related to electrified door hardware including, but not limited to, the following:

Inspect and discuss electrical roughing-in and other preparatory work performed by other trades.

Review sequence of operation for each type of electrified door hardware.

Review and finalize construction schedule and verify availability of materials, Installer's personnel, equipment, and facilities needed to make progress and avoid delays.

Review required testing, inspecting, and certifying procedures.

DELIVERY, STORAGE, AND HANDLING

Delivery:  Inventory door hardware on receipt and provide secure lock-up for door hardware delivered to Project site.  Tag each item or package separately with identification related to the final Door Hardware Schedule, and include basic installation instructions with each item or package.

COORDINATION

Coordination:  Coordinate layout and installation of recessed pivots and closers with floor construction.  Cast anchoring inserts into concrete.  Concrete, reinforcement, and formwork requirements are specified in Section 033000 - CAST-IN-PLACE CONCRETE.

Templates:  Obtain and distribute to the parties involved templates for doors, frames, and other work specified to be factory prepared for installing door hardware.  Check Shop Drawings of other work to confirm that adequate provisions are made for locating and installing door hardware to comply with indicated requirements.

WARRANTY

General Warranty:  Special warranties specified in this Article shall not deprive the Owner of other rights the Owner may have under other provisions of the Contract Documents and shall be in addition to, and run concurrent with, other warranties made by Contractor under requirements of the Contract Documents.

Special Warranty:  Written warranty, executed by manufacturer agreeing to repair or replace components of door hardware that fail in materials or workmanship within specified warranty period.  Failures include, but are not limited to, the following:

Structural failures including excessive deflection, cracking, or breakage.

Faulty operation of operators and door hardware.

Deterioration of metals, metal finishes, and other materials beyond normal weathering.

Warranty Period:  Three years from date of Substantial Completion, unless otherwise indicated.

Warranty Period for Manual Closers:  Ten years from date of Substantial Completion.

MAINTENANCE SERVICE

Maintenance Tools and Instructions:  Furnish a complete set of specialized tools and maintenance instructions as needed for the Owner's continued adjustment, maintenance, and removal and replacement of door hardware.

PRODUCTS

MANUFACTURER

Basis-of-Design, Canada Contact: K.N. CROWDER MFG. INC., 1220 Burloak Drive, Burlington, Ontario L7L 6B3. Toll Free Telephone 1-866-999-1562. Toll Free Fax 1-800-567-0123.

Basis-of-Design, USA Contact: K.N. CROWDER INC., 210 Eighth Street South Lewiston, New York 14092. Toll Free Telephone 1-866-999-1562. Toll Free Fax 1-800-567-0123.

PERFORMANCE REQUIREMENTS

Standards Compliance:

ANSI/ BHMA A156.14 – Sliding and Folding Door Hardware.

ANSI/ DHI A115.IG – Installation Guide for Doors and Hardware.

American with Disabilities Act (ADA) 2010 Standards for Accessible Design.

ANSI A117.1 Accessibility Guidelines for Building and Facilities.

All state and local accessibility standards.

Operating hardware shall be fully exposed and usable from both sides at all times.

Doors shall comply with maximum opening force of 5 lbf applied parallel to door at latch.

CROWDER CANTILEVER AND CATCH ‘N’ CLOSE SLIDING DOOR HARDWARE SYSTEM

Basis of Design: Crowder Cantilever system, CC-820 Concealed Zero Clearance Catch ‘N’ Close Hanger, side mount track system with gap in track for patient lift systems to pass through from room to room by K.N. Crowder Mfg. Inc.:

Heavy Duty Continuous Zero Clearance Catch ‘N’ Close Hanger System: Prevents bouncing and slamming of doors. 1 1/8 inch [28.6 mm] x 2 1/8 inch [54.0 mm] depth x door width slot recessed into door. Made with stainless steel and nylon wheels with precision ground ball bearings.

Stops: CC-101HD in-track adjustable stop.

Bottom Guide System: C-914 guide channel 1 inch (25.4 mm) x 29/32 inch (22.9 mm), recessed into bottom of door with C-913-2 double roller floor mounted guide.

Floor Stop: C-102HD floor mounted stop for 1 3/4 inch [44.5 mm] thick doors.

- Editor’s Note: Choose one of the following four options for door weight.

Capacity: [CC-75 for 75lbs (34kg)] [CC-1 for 150lbs (68kg)] [CC-2 for 200lbs (91kg)] [CC-3 for 300lbs (136kg)]

- Editor’s Note: Choose one of the following. Clear anodized is standard.

Complete System Finish: [Clear Anodized] [Black Anodized] [Custom Powder coated. Colour <Insert Colour>]

Door and Application:

Door: By others. See [appropriate doors sections in Division 08] <Insert Section Number> - <Insert Section title>.

Door thickness: 1 3/4 inch [44.5 mm]

Track and Fascia Components:

Side Mount Track: CC-908-Flared gap ends: 2 3/8 inch (60.1 mm) x 3 7/16 inch (87.0 mm) extruded side mount track supplied in two lengths for custom opening set-up with flared ends at gap to ease transition of sliding door across gap in track.

- Editor’s Note: Delete one of the following fascia options not required.

Sloped Fascia and End Caps: CC-990. 3 inch (76.2 mm) x 6 19/32 inch (167.2 mm) extruded fascia supplied in two lengths for to match track lengths with CC-991 (standard) & CC-992 (Notched) end caps. NOTE: For use with track and CC-990 Fascia only.

Square Fascia and End Caps: CC-980. 3 inch (76.2 mm) x 5 23/32 inch (145.0 mm) extruded fascia supplied in two lengths for to match track lengths with CC-981 (standard) & CC-982 (Notched) end caps. NOTE: For use with track and CC-980 Fascia only.

- Editor’s Note: Fill in the following required information.

Track Gap required: <Insert gap required for patient lift>

Material: Extruded aluminum, 6063 Alloy.

Application Form:

When quoting/ordering, application form must be filled out and submitted to customerservice@kncrowder.com for job specific shop drawings to be created or contact customer service 1-866-999-1562.

EXECUTION

EXAMINATION

Examine doors and frames, with Installer present, for compliance with requirements for installation tolerances, labeled fire door assembly construction, wall and floor construction, and other conditions affecting performance.

Proceed with installation only after unsatisfactory conditions have been corrected.

PREPARATION

Steel Doors and Frames:  Comply with DHI A115 series.

Surface-Applied Door Hardware:  Drill and tap doors and frames according to SDI 107.

Wood Doors:  Comply with DHI A115-W series.

INSTALLATION

Mounting Heights:  Mount door hardware units at heights indicated in following applicable publications, unless specifically indicated or required to comply with governing regulations:

Standard Steel Doors and Frames:  DHI's "Recommended Locations for Architectural Hardware for Standard Steel Doors and Frames."

Custom Steel Doors and Frames:  DHI's "Recommended Locations for Builders' Hardware for Custom Steel Doors and Frames."

Wood Doors:  DHI WDHS.3, "Recommended Locations for Architectural Hardware for Wood Flush Doors."

Installation:  Install each door hardware item to comply with manufacturer's written instructions.  Where cutting and fitting are required to install door hardware onto or into surfaces that are later to be painted or finished in another way, coordinate removal, storage, and reinstallation of surface protective trim units with finishing work.  Do not install surface-mounted items until finishes have been completed on substrates involved.

Set units level, plumb, and true to line and location.  Adjust and reinforce attachment substrates as necessary for proper installation and operation.

Drill and countersink units that are not factory prepared for anchorage fasteners.  Space fasteners and anchors according to industry standards.

ADJUSTING

Initial Adjustment:  Adjust and check each operating item of door hardware and each door to ensure proper operation or function of every unit.  Replace units that cannot be adjusted to operate as intended.  Adjust door control devices to compensate for final operation of heating and ventilating equipment and to comply with referenced accessibility requirements.

Six-Month Adjustment:  Approximately six months after date of Substantial Completion, Installer shall perform the following:

Examine and readjust each item of door hardware as necessary to ensure function of doors, door hardware, and electrified door hardware.

Consult with and instruct the Owner's personnel on recommended maintenance procedures.

Replace door hardware items that have deteriorated or failed due to faulty design, materials, or installation of door hardware units.

CLEANING AND PROTECTION

Clean adjacent surfaces soiled by door hardware installation.

Clean operating items as necessary to restore proper function and finish.

Provide final protection and maintain conditions that ensure door hardware is without damage or deterioration at time of Substantial Completion.

HARDWARE SET SCHEDULE

Refer to the Drawings.

END OF SECTION